# Assignment-3: CRC Problems

Name: Manish Kumar

Roll No: 2503b09903

## Task 1

Problem 01:

A bit stream 1101011011 is transmitted using the standard CRC method. The generator polynomial is x⁴ + x + 1. What is the actual bit string transmitted?

Solution:

Step 1: Write the generator polynomial in binary form.

G(x) = x⁴ + x + 1  ⇒  Generator bits = 10011

Step 2: Take the data bits and append 4 zeros (degree of generator is 4):

Data bits: 1101011011

Appended bits: 1101011011 0000

Step 3: Perform modulo-2 division of 11010110110000 by 10011.
The remainder obtained is: 1110

Step 4: Append the remainder to the original data bits to form the transmitted frame.

Transmitted bit string = 1101011011 1110

Therefore, the actual bit string transmitted is:

11010110111110

## Task 2

Problem 02:

A bit stream 10011101 is transmitted using the standard CRC method. The generator polynomial is x³ + 1.
1) What is the actual bit string transmitted?
2) Suppose the third bit from the left is inverted during transmission. How will the receiver detect this error?

Solution:

Part (1): Finding the transmitted bit string

Step 1: Write the generator polynomial in binary form.

G(x) = x³ + 1  ⇒  Generator bits = 1001

Step 2: Append 3 zeros to the data bits (degree of generator is 3).

Data bits: 10011101

Appended bits: 10011101 000

Step 3: Perform modulo-2 division of 10011101000 by 1001.
The remainder obtained is: 100

Step 4: Append the remainder to the original data bits.

Transmitted bit string = 10011101 100

Therefore, the actual bit string transmitted is:

10011101100

Part (2): Error detection at the receiver

Given that the third bit from the left is inverted during transmission.

Correct transmitted frame      : 10011101100

Third bit from left inverted ⇒ 10111101100

At the receiver:

The receiver divides the received bit string by the same generator polynomial (1001) using modulo-2 division.
• For the correct (no-error) frame 10011101100, the remainder is 000.
• For the erroneous frame 10111101100, the remainder is 100 (non-zero).

If the remainder is non-zero, the receiver concludes that an error has occurred during transmission.
Hence, when the third bit is inverted, the non-zero remainder allows the receiver to detect the error.